NUTANIX KUBERNETES PLATFORM 2.18 · PART 2 (UBUNTU EDITION) Bastion VM Build Guide Offline package staging and installation — Ubuntu Server 24.04 LTS on Nutanix AHV Companion to: NKP 2.18 Air-Gapped Deployment & Bastion Runbook rev 1.0 (§7), the Darksite Field Reference, and the RHEL 9 edition of this guide

| Item | Value |
| --- | --- |
| Prepared for | Varun Selvaraj — Staff Consulting Architect, AI Specialist |
| Purpose | Step-by-step build of the air-gapped NKP bastion on Ubuntu Server: which packages are needed, how to download them on an Internet-connected machine, how to build a signed offline apt repository, and how to install on a bastion VM with no Internet — with the verification that proves it before travel. |
| Scope | Connected downloader → signed offline apt repository → transfer kit → bastion VM in Prism Central → Ubuntu offline preparation → Docker, CLIs, trust, hardening → Gate 2. NKP bundle push and cluster creation: Field Reference §4. |
| Baseline | Ubuntu Server 24.04 LTS (noble) amd64 — same OS family as the NKP 2.18 Ubuntu node image; 22.04 delta in §7.1. Docker Engine ≥ 27.4 from Docker's Ubuntu repository (Podman alternative in §7.2). External enterprise registry (Harbor-on-bastion notes in §4.11). NKP 2.18.x. |
| Validation | Repository build (§2.5) and the offline install from it (§4.3–4.4) executed end to end on Ubuntu 24.04.4 on 2026-09-03; Docker repository steps follow the official Docker documentation for Ubuntu. |
| Revision | 1.0 — 2026-09-03 |
| Classification | Customer Confidential — Field working copy |

What is different from the RHEL edition — and the one rule that still applies Ubuntu has no "DVD with the whole distribution": the live-server ISO carries only the base system, so on Ubuntu the set of `.deb` files you download on the connected machine is the bastion's repository. The rule is therefore stricter: build the downloader from the same ISO, with the same installer choices, with its network unplugged during installation (so subiquity cannot pull updates), and never apt upgrade it. apt-get install --download-only then fetches exactly what the identically-built bastion is missing. Then build, hash, sign, and prove the kit on a second network-less VM before travel.

Contents

1. Design baseline and end state	3

2. Part A — Connected downloader VM and the transfer kit	4

2.1 Build the downloader exactly like the bastion	4

2.2 Download and verify the Ubuntu Server ISO	4

2.3 Download the OS package set (and, optionally, security updates)	4

2.4 Docker Engine `.deb` set from Docker's Ubuntu repository (pinned)	5

2.5 Build and sign the offline apt repository (`nkp-offline`)	6

2.6 CLI binaries with checksums	6

2.7 Optional — Harbor offline installer and validation images	7

2.8 Configuration files to carry (create now, install in Part C)	7

2.9 Assemble, hash, and test the kit on a disposable offline VM	9

3. Part B — Create the bastion VM in Prism Central	10

3.1 Upload the ISO to the image service	10

3.2 VM settings	10

3.3 Install Ubuntu Server from the console (subiquity)	10

3.4 First boot — netplan static network, hostname, SSH key	11

4. Part C — Offline OS preparation on the bastion	13

4.1 Data disks — LVM + XFS + fstab	13

4.2 Bring the kit onto the bastion and verify it	13

4.3 Configure apt: the signed local repository and nothing else	14

4.4 Install the OS package set (and the carried security updates)	14

4.5 Time, DNS and hosts	15

4.6 Certificate trust — OS, Docker, and the PC bundle for NKP	15

4.7 Kernel modules, sysctl, limits	15

4.8 Docker Engine from the local repository (pinned and held)	16

4.9 CLI binaries — kubectl, helm, yq, nkp	17

4.10 Security baseline — AppArmor, ufw, SSH, proxy hygiene	17

4.11 Optional — Harbor on the bastion (key points only; flow in runbook §8.4)	17

4.12 Load the bootstrap image, snapshot, hand over to Gate 2	18

5. Gate 2 verification — bastion ready	19

6. Package reference and RHEL ↔ Ubuntu equivalents	20

6.1 If you have the RHEL edition open next to this one	21

7. Variants and troubleshooting	21

7.1 Ubuntu 22.04 LTS delta	21

7.2 Podman instead of Docker Engine	22

7.3 Bastion from the NKP Ubuntu node image (not recommended)	22

7.4 Troubleshooting	22

7.5 Keeping the bastion patched later	23

# 1. Design baseline and end state

| Decision | Value used in this guide | Why / alternative |
| --- | --- | --- |
| Bastion OS | Ubuntu Server 24.04.x LTS amd64, "Ubuntu Server" (not minimized) install from the customer-approved ISO | Same family as the NKP Ubuntu 24.04 node image; five years of LTS. 22.04 works with the §7.1 deltas. |
| Container runtime | Docker Engine (docker-ce) ≥ 27.4.0 from Docker's Ubuntu repository, pinned and held | Ubuntu's own docker.io package is older than NKP's 27.4 minimum; the Docker snap is unsuitable for servers. Podman 4.9 from universe is the alternative (§7.2). |
| Registry | Customer enterprise registry reachable on TCP 443 | Harbor-on-bastion only when no registry exists (§4.11, runbook §8.4). |
| Bastion sizing | 8 vCPU, 32 GiB RAM; disks: 120 GiB OS, 300 GiB /opt/nkp, 250 GiB /var/lib/docker (+ 500 GiB–1.5 TB /srv/registry for Harbor) | Bundle ~29 GB + extraction ~29 GB + bootstrap image + logs; Docker needs 8 GiB RAM for KinD. |
| Network | One NIC on the restricted management subnet, static IPv4 via netplan, internal DNS/NTP, zero egress | Runbook §3.2 and §5. |
| Administrator | nkpadmin, SSH public-key only, sudo + docker groups; root has no password (Ubuntu default), console recovery via sudo -i | Runbook §7.2. |
| Package source on the bastion | Exactly one apt source: the signed flat repository file:/opt/nkp/artifacts/debs ./ built in Part A | No online source is ever defined; Ubuntu's ubuntu.sources, snapd, unattended-upgrades, apt timers, Pro client and cloud-init are disabled. |
| MAC / firewall | AppArmor enforcing (default); ufw with SSH only | Ubuntu equivalents of SELinux / firewalld. |

End state after Part C: a hardened Ubuntu 24.04 VM with AppArmor enforcing and ufw on, chrony synchronised to internal NTP, customer CA trusted by the OS and Docker, Docker Engine running with a non-overlapping address pool, kubectl/helm/nkp installed and version-checked, the NKP bootstrap image loaded, ≥ 60 GB free under /opt/nkp, and proof that public egress fails. That is runbook Gate 2.

# 2. Part A — Connected downloader VM and the transfer kit

## 2.1 Build the downloader exactly like the bastion

- Any hypervisor with Internet access: 4 vCPU, 8 GiB RAM, 150 GB disk. Install from the same Ubuntu 24.04.x live-server ISO (§2.2) with the same installer choices you will make in §3.3 (edition, storage type, OpenSSH server, no snaps).

- Install with the NIC disconnected (or the vNIC unplugged in the hypervisor). With network available, subiquity downloads a newer installer and applies security updates at the end of the install — the base would then differ from the bastion's. Connect the network only after the first boot.

- Never run apt upgrade on the downloader. apt update is required and harmless (it only refreshes the index).

- Keep /var/log/installer/autoinstall-user-data — subiquity writes the autoinstall file that reproduces this exact installation (§3.3).

Downloader — baseline

# On the downloader, after first boot with the network now connected sudo apt-get update sudo apt-get install -y dpkg-dev apt-utils gnupg skopeo jq curl xorriso rsync tree mkdir -p ~/kit/{os,debs/pool,bin,nkp,harbor,images/validation,pki,config,docs} sudo apt-get clean # empty /var/cache/apt/archives so it only ever holds kit packages from now on lsb_release -ds; uname -r # record: must match the bastion after installation

## 2.2 Download and verify the Ubuntu Server ISO

Downloader — ISO into the kit

cd ~/kit/os UBU="24.04.<x>" # the point release you standardise on for downloader, bastion and test VM curl -fsSLO "https://releases.ubuntu.com/24.04/ubuntu-${UBU}-live-server-amd64.iso" curl -fsSLO "https://releases.ubuntu.com/24.04/SHA256SUMS" curl -fsSLO "https://releases.ubuntu.com/24.04/SHA256SUMS.gpg" # Ubuntu CD image signing keys (per ubuntu.com "How to verify your Ubuntu download") gpg --keyid-format long --keyserver hkp://keyserver.ubuntu.com --recv-keys 0x46181433FBB75451 0xD94AA3F0EFE21092 gpg --keyid-format long --verify SHA256SUMS.gpg SHA256SUMS # "Good signature from Ubuntu CD Image Automatic Signing Key" sha256sum -c SHA256SUMS --ignore-missing # ubuntu-24.04.x-live-server-amd64.iso: OK cp /usr/share/keyrings/ubuntu-archive-keyring.gpg ~/kit/config/ # reference copy of the Ubuntu archive key

The live-server ISO (~2.5–3 GB) is the installer plus the base system only. It can be added on the bastion as a fallback apt source for base packages (§4.3), but it does not contain the tool set — that comes from the repository you build in §2.5.

## 2.3 Download the OS package set (and, optionally, security updates)

apt-get install --download-only resolves against the downloader's installed base and downloads every package the bastion (same base) will need, including dependency upgrades of base libraries. Nothing is installed on the downloader. Everything lands in /var/cache/apt/archives, which you emptied in §2.1.

Downloader — OS packages

sudo apt-get update sudo apt-get install -y --download-only \ bash-completion vim tmux git tar unzip zip xz-utils bzip2 rsync wget curl jq python3 tree \ dnsutils netcat-openbsd iputils-ping iputils-arping traceroute tcpdump net-tools socat ethtool lsof strace sysstat iotop \ chrony openssl ca-certificates gnupg \ lvm2 xfsprogs \ ufw apparmor-utils auditd \ skopeo dpkg-dev apt-utils sosreport # Optional but recommended: carry the security updates for the base system. # Because the bastion has the identical base, "apt-get upgrade" on it will consume exactly these files. sudo apt-get -y --download-only upgrade ls /var/cache/apt/archives/*.deb ¦ wc -l; du -sh /var/cache/apt/archives

If the downloader base cannot be kept identical Fall back to the full recursive closure, which downloads every dependency whether or not it is already installed (larger, but base-independent). Use it for the whole list or for a single package that failed on the test VM: apt-get download $(apt-cache depends --recurse --no-recommends --no-suggests --no-conflicts --no-breaks --no-replaces --no-enhances <packages> ¦ grep "^\w" ¦ sort -u) — run inside ~/kit/debs/pool. Recommended packages (apt installs them by default) are then missing; add them explicitly.

## 2.4 Docker Engine `.deb` set from Docker's Ubuntu repository (pinned)

Downloader — Docker CE

sudo install -m 0755 -d /etc/apt/keyrings sudo curl -fsSL https://download.docker.com/linux/ubuntu/gpg -o /etc/apt/keyrings/docker.asc sudo chmod a+r /etc/apt/keyrings/docker.asc gpg --show-keys /etc/apt/keyrings/docker.asc # fingerprint must be 9DC8 5822 9FC7 DD38 854A E2D8 8D81 803C 0EBF CD88 echo "deb [arch=$(dpkg --print-architecture) signed-by=/etc/apt/keyrings/docker.asc] https://download.docker.com/linux/ubuntu \ $(. /etc/os-release && echo "${UBUNTU_CODENAME:-$VERSION_CODENAME}") stable" ¦ sudo tee /etc/apt/sources.list.d/docker.list >/dev/null sudo apt-get update # List and pin ONE version >= 27.4.0. Record the exact string in the kit manifest. apt-cache madison docker-ce ¦ awk '{print $3}' ¦ head DOCKER_VER='5:<major.minor.patch>-1~ubuntu.24.04~noble' sudo apt-get install -y --download-only \ docker-ce="${DOCKER_VER}" docker-ce-cli="${DOCKER_VER}" containerd.io docker-buildx-plugin docker-compose-plugin echo "DOCKER_VER=${DOCKER_VER}" >> ~/kit/config/VERSIONS.txt cp /etc/apt/keyrings/docker.asc ~/kit/config/docker.asc # reference only - the kit repository is signed with the kit key (2.5)

## 2.5 Build and sign the offline apt repository (`nkp-offline`)

A flat repository: all .deb files in pool/, a Packages index, a Release file with hashes, and a detached/clear-signed signature from a kit signing key that lives only on the downloader. The bastion trusts the exported public key via signed-by, so apt verifies the index exactly as it would for an online mirror. The .deb hashes are inside Packages, so every package is integrity-checked on install.

Downloader — signed flat repository (commands verified on 24.04.4)

sudo cp /var/cache/apt/archives/*.deb ~/kit/debs/pool/ && sudo chown -R "$USER":"$USER" ~/kit/debs cd ~/kit/debs dpkg-scanpackages --multiversion pool /dev/null > Packages && gzip -9kf Packages apt-ftparchive release . > Release # Kit signing key - create once; the private key never leaves the downloader gpg --batch --passphrase "" --quick-gen-key "NKP Kit Signing <nkp-kit@customer.local>" ed25519 sign 2y KEYID=$(gpg --list-keys --with-colons nkp-kit@customer.local ¦ awk -F: '/^pub/{print $5}'); echo "$KEYID" gpg --batch --yes --default-key "$KEYID" --clearsign -o InRelease Release gpg --batch --yes --default-key "$KEYID" -abs -o Release.gpg Release gpg --export "$KEYID" > nkp-kit-archive.gpg # binary keyring, goes to /etc/apt/keyrings on the bastion ls -la ~/kit/debs ¦ grep -v pool; ls ~/kit/debs/pool ¦ wc -l # Self-test on the downloader using ONLY this repository (no system change; _apt must be able to read the path) sudo cp -r ~/kit/debs /opt/kit-test-debs && sudo chmod -R o+rX /opt/kit-test-debs echo "deb [signed-by=/opt/kit-test-debs/nkp-kit-archive.gpg] file:/opt/kit-test-debs ./" ¦ sudo tee /tmp/nkp-offline.list APTX="-o Dir::Etc::SourceList=/tmp/nkp-offline.list -o Dir::Etc::SourceParts=/dev/null" sudo apt-get $APTX update # only "file:/opt/kit-test-debs ./ InRelease" apt-cache $APTX policy docker-ce chrony skopeo ¦ grep -E "Candidate¦file:" sudo rm -rf /opt/kit-test-debs /tmp/nkp-offline.list

Re-building after adding packages Any time a .deb is added to pool/ (a forgotten tool, a newer Docker pin, the errata set), re-run the four index commands (dpkg-scanpackages, gzip, apt-ftparchive release, the two gpg signatures). The dpkg-dev and apt-utils packages are in the kit, so this can also be done on the bastion with a re-signing key if the customer allows a key there; otherwise rebuild on the downloader and re-carry only debs/.

## 2.6 CLI binaries with checksums

Downloader — kubectl, helm, yq

cd ~/kit/bin # kubectl - match the cluster minor (NKP 2.18 = Kubernetes 1.35) KVER=$(curl -fsSL https://dl.k8s.io/release/stable-1.35.txt); echo "$KVER" curl -fsSLO "https://dl.k8s.io/release/${KVER}/bin/linux/amd64/kubectl" curl -fsSLO "https://dl.k8s.io/release/${KVER}/bin/linux/amd64/kubectl.sha256" echo "$(cat kubectl.sha256) kubectl" ¦ sha256sum --check # helm - 3.x ONLY (Helm 4 breaks NDK). Newest v3 tag from github.com/helm/helm/releases HVER="v3.<x>.<y>" curl -fsSLO "https://get.helm.sh/helm-${HVER}-linux-amd64.tar.gz" curl -fsSLO "https://get.helm.sh/helm-${HVER}-linux-amd64.tar.gz.sha256sum" sha256sum --check "helm-${HVER}-linux-amd64.tar.gz.sha256sum" # yq (optional) YQVER="v4.<x>.<y>" curl -fsSLo yq "https://github.com/mikefarah/yq/releases/download/${YQVER}/yq_linux_amd64" && sha256sum yq ¦ tee yq.sha256 # nkp is inside the air-gapped bundle (nkp-v2.18.x/cli/nkp) - nothing to download here printf "kubectl %s\nhelm %s\nyq %s\n" "$KVER" "$HVER" "$YQVER" >> ~/kit/config/VERSIONS.txt

## 2.7 Optional — Harbor offline installer and validation images

Downloader — Harbor and validation images

# Harbor (only without an enterprise registry) - newest 2.x; NKP supports Harbor 2.1.1+ cd ~/kit/harbor && HARBOR="v2.<x>.<y>" curl -fsSLO "https://github.com/goharbor/harbor/releases/download/${HARBOR}/harbor-offline-installer-${HARBOR}.tgz" curl -fsSLO "https://github.com/goharbor/harbor/releases/download/${HARBOR}/harbor-offline-installer-${HARBOR}.tgz.asc" # import the Harbor release signing key as documented at goharbor.io/docs, then: gpg --verify "harbor-offline-installer-${HARBOR}.tgz.asc" "harbor-offline-installer-${HARBOR}.tgz" # Smoke-test images as docker-archives (not in the NKP bundle) cd ~/kit/images/validation skopeo copy docker://docker.io/library/busybox:1.37 docker-archive:./busybox-1.37.tar:busybox:1.37 skopeo copy docker://docker.io/hashicorp/http-echo:1.0 docker-archive:./http-echo-1.0.tar:http-echo:1.0 # confirm the tag for i in library/busybox:1.37 hashicorp/http-echo:1.0; do skopeo inspect "docker://docker.io/$i" ¦ jq -r "\"$i \" + .Digest"; done ¦ tee DIGESTS.txt

## 2.8 Configuration files to carry (create now, install in Part C)

~/kit/config/nkp-offline.list → /etc/apt/sources.list.d/nkp-offline.list

deb [signed-by=/etc/apt/keyrings/nkp-kit.gpg] file:/opt/nkp/artifacts/debs ./ # Fallback base-system source from the mounted ISO (optional; contains only what the installer shipped): # deb [trusted=yes] file:/mnt/ubuntu noble main restricted

~/kit/config/20auto-upgrades → /etc/apt/apt.conf.d/20auto-upgrades

APT::Periodic::Update-Package-Lists "0"; APT::Periodic::Download-Upgradeable-Packages "0"; APT::Periodic::AutocleanInterval "0"; APT::Periodic::Unattended-Upgrade "0";

~/kit/config/50-nkp-static.yaml → /etc/netplan/50-nkp-static.yaml (mode 0600)

network: version: 2 ethernets: ens3: # confirm with: ip -br link dhcp4: false dhcp6: false accept-ra: false addresses: [<BASTION_IP>/<PREFIX>] routes: - to: default via: <GATEWAY> nameservers: addresses: [<DNS1>, <DNS2>] search: [<customer.local>]

~/kit/config/docker-daemon.json (change 172.30.0.0/16 to a range that overlaps nothing at the customer - KinD builds its "kind" bridge from this pool)

{ "default-address-pools": [ { "base": "172.30.0.0/16", "size": 24 } ], "log-driver": "json-file", "log-opts": { "max-size": "50m", "max-file": "5" }, "storage-driver": "overlay2", "live-restore": false }

~/kit/config/nkp-bastion-modules.conf

# /etc/modules-load.d/nkp-bastion.conf overlay br_netfilter

~/kit/config/99-nkp-bastion.conf (inotify values are the documented KinD fix for "too many open files")

# /etc/sysctl.d/99-nkp-bastion.conf net.ipv4.ip_forward = 1 net.bridge.bridge-nf-call-iptables = 1 net.bridge.bridge-nf-call-ip6tables = 1 fs.inotify.max_user_watches = 524288 fs.inotify.max_user_instances = 512 fs.file-max = 2097152

~/kit/config/nkp-bastion-limits.conf

# /etc/security/limits.d/nkp-bastion.conf * soft nofile 65536 * hard nofile 131072 nkpadmin soft nproc 16384 nkpadmin hard nproc 32768

~/kit/config/10-nkp-sshd.conf

# /etc/ssh/sshd_config.d/10-nkp-sshd.conf - the "10-" prefix is deliberate: sshd takes the FIRST value it reads, # and subiquity leaves /etc/ssh/sshd_config.d/50-cloud-init.conf with "PasswordAuthentication yes" (removed in 4.10) PermitRootLogin no PasswordAuthentication no KbdInteractiveAuthentication no PubkeyAuthentication yes X11Forwarding no MaxAuthTries 4 ClientAliveInterval 300 ClientAliveCountMax 2

~/kit/config/nkp.sources

# /etc/chrony/sources.d/nkp.sources (Ubuntu chrony.conf already contains "sourcedir /etc/chrony/sources.d") server <NTP1_IP_OR_FQDN> iburst server <NTP2_IP_OR_FQDN> iburst

## 2.9 Assemble, hash, and test the kit on a disposable offline VM

Downloader — kit assembly

cd ~/kit cp ~/downloads/nkp-air-gapped-bundle_v2.18.x_linux_amd64.tar.gz ~/downloads/nkp-ubuntu-24.04-*-1.35.2-*.qcow2 ~/kit/nkp/ # or the Rocky node image sudo cp /var/log/installer/autoinstall-user-data ~/kit/config/bastion-autoinstall-user-data && sudo chown "$USER" ~/kit/config/bastion-autoinstall-user-data tree -L 2 ~/kit ¦ tee ~/kit/KIT-TREE.txt find . -type f ! -name SHA256SUMS -print0 ¦ sort -z ¦ xargs -0 sha256sum > SHA256SUMS sha256sum -c SHA256SUMS ¦ grep -vc ": OK$"; du -sh ~/kit # Optional: single ISO of the kit for CD-ROM attachment via the Prism Central image service (-iso-level 3: files > 4 GB) xorriso -as mkisofs -iso-level 3 -R -J -joliet-long -V NKPKIT -o ~/nkp-kit-$(date +%Y%m%d).iso ~/kit && sha256sum ~/nkp-kit-*.iso

Pre-travel test that is not optional Create a second VM from the same ISO with no network, using the same installer choices as §3.3 (or the autoinstall file). Attach the kit (ISO or copied directory). Run Part C exactly as written — including apt-get upgrade from the kit if you carry updates — then docker load the NKP bootstrap image and, with no Prism Central involved, nkp create bootstrap followed by nkp delete bootstrap. On Ubuntu this test is the only proof that the .deb set is complete: a missing dependency shows up here as "Depends: … but it is not going to be installed", and here you can still fix it. Keep the console log in ~/kit/docs/kit-test-<date>.log and carry it.

# 3. Part B — Create the bastion VM in Prism Central

## 3.1 Upload the ISO to the image service

- Prism Central → Infrastructure → Images → Add Image. Source: *Image File* or *URL* (internal HTTP server inside the secure zone). Type ISO, name ubuntu-24.04.<x>-live-server-amd64, description = vendor filename + SHA-256. Wait until *Active* on the target cluster.

- Optional: upload the kit ISO from §2.9 (nkp-kit-<date>) the same way, so the kit reaches the bastion as a second CD-ROM without a network copy.

- Optional: a tiny cidata ISO with user-data (the autoinstall file) and an empty meta-data for unattended installation (§3.3).

## 3.2 VM settings

| Setting | Value | Notes |
| --- | --- | --- |
| Name / description | nkp-bastion-01 — "NKP 2.18 air-gapped bastion, owner, change ref" | Customer naming standard wins. |
| vCPU / cores / memory | 8 vCPU × 1 core, 32 GiB | 12–16 vCPU / 48–64 GiB with Harbor co-hosted; no host memory overcommit. |
| Boot configuration | UEFI (Secure Boot off unless mandated) — or Legacy BIOS | Ubuntu 24.04 supports both; UEFI needs the ESP in §3.3. |
| CD-ROM 1 | Attach ubuntu-24.04.<x>-live-server-amd64 | Keep attached; usable as a fallback apt source (/dev/sr0). |
| CD-ROM 2 (optional) | nkp-kit-<date> ISO | /dev/sr1 in the guest. |
| Disk 1 (SCSI) | 120 GiB — OS | VirtIO-SCSI/virtio-net drivers are in the Ubuntu kernel. |
| Disk 2 (SCSI) | 300 GiB — /opt/nkp | Bundle, extraction, artefacts, evidence, logs. |
| Disk 3 (SCSI) | 250 GiB — /var/lib/docker | Bootstrap image, KinD, Harbor containers. |
| Disk 4 (SCSI, Harbor only) | 500 GiB – 1.5 TB — /srv/registry | Protect with backup; separate VG. |
| Storage container | Platform/management container approved by the customer | Not the NKP CSI container unless the customer says so. |
| NIC | Restricted management subnet; if AHV-IPAM managed, assign the reserved static IP on the NIC | Prevents a DHCP lease colliding with the netplan static address. |
| Categories | Environment, owner, application, backup | Runbook §7.2. |
| Boot order | CD-ROM first for installation, then Disk 1 first | Set explicitly in *Boot Configuration* after the install. |
| Guest tools | Nutanix Guest Tools optional | Not required for Linux. |

## 3.3 Install Ubuntu Server from the console (subiquity)

| Installer screen | Choice |
| --- | --- |
| Language / keyboard | English; customer layout. If offered "Update to the new installer" (only with network), it cannot happen on the bastion — and must be declined on the downloader. |
| Type of installation | Ubuntu Server (not *minimized*) — identical to the downloader. Do not tick third-party drivers. |
| Network connections | Edit the NIC → IPv4 *Manual*: subnet, address, gateway, name servers, search domain. IPv6: *Disabled*. (No DHCP on the management subnet is assumed.) |
| Proxy | Blank. |
| Ubuntu archive mirror | Leave the default; the installer reports the mirror as unreachable — choose Continue without updates / *Continue*. On the bastion this is expected: the base system installs from the ISO pool. |
| Guided storage configuration | Custom storage layout on Disk 1 only (leave Disks 2–4 untouched). Suggested for 120 GiB, UEFI: /boot/efi 1 GiB (fat32), /boot 2 GiB (ext4), LVM VG vg_sys: / 50 GiB, /var 30 GiB, /var/log 10 GiB, /var/log/audit 5 GiB, /home 10 GiB, /tmp 5 GiB, swap 4 GiB (or none — NKP/KinD does not need swap). Adjust to the customer CIS baseline if mandated. Gotcha: the *guided* LVM option does not always allocate the whole disk to ubuntu-lv; if you use it, run lvextend -l +100%FREE /dev/ubuntu-vg/ubuntu-lv && resize2fs /dev/ubuntu-vg/ubuntu-lv after the first boot. |
| Profile configuration | Your name, server name nkp-bastion-01, username nkpadmin, password (used once for ssh-copy-id; password login is disabled in §4.10). |
| Ubuntu Pro | Skip for now. |
| SSH configuration | Install OpenSSH server = yes; *Import SSH identity* = No. |
| Featured server snaps | Select none. |
| Installing system | The final "downloading and installing security updates" step fails or is skipped without network — expected on the bastion; must be prevented on the downloader by unplugging its NIC (§2.1). |
| Autoinstall alternative | Build a cidata ISO on the downloader: xorriso -as mkisofs -V cidata -J -R -o cidata.iso user-data meta-data where user-data is the copied autoinstall-user-data (review the identity/storage sections) and meta-data is an empty file. Attach it as CD-ROM 3, boot the Ubuntu ISO, press e on the GRUB entry and append autoinstall to the linux line. |

## 3.4 First boot — netplan static network, hostname, SSH key

Bastion — first boot

# Prism console session (do not rely on SSH until this is proven) sudo -i ip -br link # NIC name, e.g. ens3 - must match the netplan file touch /etc/cloud/cloud-init.disabled # cloud-init has no datasource on AHV; stop it touching netplan mkdir -p /root/netplan.disabled && mv /etc/netplan/50-cloud-init.yaml /root/netplan.disabled/ 2>/dev/null ¦¦ true install -m 0600 /path/to/kit/config/50-nkp-static.yaml /etc/netplan/50-nkp-static.yaml # or type it from 2.8 netplan generate && netplan try # press ENTER within 120 s to keep, otherwise it reverts netplan apply hostnamectl set-hostname nkp-bastion-01.customer.local ip -br address; ip route; resolvectl status ¦ grep -E "Link¦DNS Servers¦DNS Domain" ping -c 3 <GATEWAY>; getent ahostsv4 pc.customer.local; getent ahostsv4 registry.customer.local # From the jump host: install the admin public key while password auth still works ssh-copy-id -i ~/.ssh/nkp_admin.pub nkpadmin@<BASTION_IP> ssh -i ~/.ssh/nkp_admin nkpadmin@<BASTION_IP> "sudo -n true && echo sudo-ok"

# 4. Part C — Offline OS preparation on the bastion

Order matters: data disks first (the kit lands on /opt/nkp), then the kit, then the apt source, then packages. All commands as nkpadmin with sudo; run them inside tmux once it is installed (§4.4), from the console before that. Set export DEBIAN_FRONTEND=noninteractive NEEDRESTART_MODE=a in the session to suppress interactive prompts from needrestart.

## 4.1 Data disks — LVM + XFS + fstab

Bastion — data disks

lsblk -o NAME,SIZE,TYPE,FSTYPE,MOUNTPOINTS,SERIAL # identify the empty data disks by SIZE sudo wipefs -n /dev/sdb; sudo wipefs -n /dev/sdc # must print nothing sudo pvcreate /dev/sdb /dev/sdc sudo vgcreate vg_nkp /dev/sdb && sudo vgcreate vg_docker /dev/sdc sudo lvcreate -l 100%FREE -n lv_nkp vg_nkp && sudo lvcreate -l 100%FREE -n lv_docker vg_docker sudo mkfs.xfs /dev/vg_nkp/lv_nkp && sudo mkfs.xfs /dev/vg_docker/lv_docker # xfsprogs is on the base install; else use mkfs.ext4 sudo mkdir -p /opt/nkp /var/lib/docker for pair in "vg_nkp/lv_nkp:/opt/nkp" "vg_docker/lv_docker:/var/lib/docker"; do dev="/dev/${pair%%:*}"; mp="${pair##*:}"; uuid=$(sudo blkid -s UUID -o value "$dev") echo "UUID=${uuid} ${mp} xfs defaults,nofail 0 0" ¦ sudo tee -a /etc/fstab done sudo systemctl daemon-reload && sudo mount -a && df -hT /opt/nkp /var/lib/docker # Harbor only: repeat for /dev/sdd -> vg_registry/lv_registry -> /srv/registry

## 4.2 Bring the kit onto the bastion and verify it

| Option | How | When |
| --- | --- | --- |
| A — rsync from the customer jump host | Media read on the jump host (portable SSD, exFAT/ext4); rsync -avh --progress /media/kit/ nkpadmin@<BASTION_IP>:/opt/nkp/quarantine/ | Default at most sites. |
| B — kit ISO as CD-ROM 2 | Upload nkp-kit-<date>.iso to the image service, attach, sudo mount -o ro /dev/sr1 /mnt/kit, copy to quarantine | No network copy path, or malware scanning happens at image upload. |
| C — transfer gateway / data diode | Customer-specific landing zone, then rsync from there | Regulated sites. |

Bastion — kit intake

sudo install -d -m 0755 /opt/nkp && sudo install -d -m 0750 -o nkpadmin -g nkpadmin /opt/nkp/{quarantine,config,pki,evidence,logs} sudo install -d -m 0755 -o nkpadmin -g nkpadmin /opt/nkp/artifacts # 0755: the _apt sandbox user must traverse it (4.3) # ... copy the kit into /opt/nkp/quarantine using option A, B or C ... cd /opt/nkp/quarantine && sha256sum -c SHA256SUMS ¦ grep -v ": OK$" ¦¦ echo "ALL FILES OK" # Customer malware scan / approval here (chain of custody, runbook §6.7). Then release: rsync -a --remove-source-files /opt/nkp/quarantine/ /opt/nkp/artifacts/ && find /opt/nkp/quarantine -type d -empty -delete ls /opt/nkp/artifacts # bin config debs docs harbor images nkp os pki KIT-TREE.txt SHA256SUMS

## 4.3 Configure apt: the signed local repository and nothing else

Bastion — apt (verified on 24.04.4)

# Trust the kit signing key and define the single source sudo install -d -m 0755 /etc/apt/keyrings sudo install -m 0644 /opt/nkp/artifacts/debs/nkp-kit-archive.gpg /etc/apt/keyrings/nkp-kit.gpg sudo install -m 0644 /opt/nkp/artifacts/config/nkp-offline.list /etc/apt/sources.list.d/nkp-offline.list # Remove every online source: 24.04 uses deb822 ubuntu.sources; older installs use sources.list; the installer adds a cdrom line sudo mkdir -p /root/apt.disabled sudo mv /etc/apt/sources.list.d/ubuntu.sources /root/apt.disabled/ 2>/dev/null ¦¦ true sudo sed -i "s/^deb /# deb /" /etc/apt/sources.list 2>/dev/null ¦¦ true ls /etc/apt/sources.list.d/ # only nkp-offline.list # Let the _apt sandbox user read the repository (otherwise apt warns "Download is performed unsandboxed as root") sudo chmod -R o+rX /opt/nkp/artifacts/debs sudo apt-get update # expect exactly: Get:1 file:/opt/nkp/artifacts/debs ./ InRelease ... Packages apt-cache policy docker-ce chrony skopeo ¦ grep -E "Candidate¦file:" # Silence everything that phones home sudo install -m 0644 /opt/nkp/artifacts/config/20auto-upgrades /etc/apt/apt.conf.d/20auto-upgrades sudo systemctl disable --now apt-daily.timer apt-daily-upgrade.timer motd-news.timer ua-timer.timer 2>/dev/null ¦¦ true sudo systemctl disable --now snapd.socket snapd.service snapd.seeded.service 2>/dev/null; sudo systemctl mask snapd.service snapd.socket 2>/dev/null ¦¦ true sudo pro config set apt_news=false 2>/dev/null ¦¦ true sudo touch /etc/cloud/cloud-init.disabled # Optional fallback source: the mounted installer ISO (base system packages only) # sudo mkdir -p /mnt/ubuntu && sudo mount -o ro /dev/sr0 /mnt/ubuntu # echo "deb [trusted=yes] file:/mnt/ubuntu noble main restricted" ¦ sudo tee -a /etc/apt/sources.list.d/nkp-offline.list && sudo apt-get update

## 4.4 Install the OS package set (and the carried security updates)

Bastion — OS packages

export DEBIAN_FRONTEND=noninteractive NEEDRESTART_MODE=a sudo -E apt-get install -y \ bash-completion vim tmux git tar unzip zip xz-utils bzip2 rsync wget curl jq python3 tree \ dnsutils netcat-openbsd iputils-ping iputils-arping traceroute tcpdump net-tools socat ethtool lsof strace sysstat iotop \ chrony openssl ca-certificates gnupg \ lvm2 xfsprogs \ ufw apparmor-utils auditd \ skopeo dpkg-dev apt-utils sosreport # chrony replaces systemd-timesyncd automatically (conflicting package) # Only if the kit carries the security updates from 2.3 - installs the same files "apt upgrade" would fetch online sudo -E apt-get -y upgrade && [ -f /var/run/reboot-required ] && echo "REBOOT REQUIRED (kernel/libc updated)" tmux new -s bastion-build # continue inside tmux from here

## 4.5 Time, DNS and hosts

Bastion — time and names

sudo timedatectl set-timezone UTC sudo sed -i "s/^pool /# pool /" /etc/chrony/chrony.conf # drop Ubuntu's public pools sudo install -m 0644 /opt/nkp/artifacts/config/nkp.sources /etc/chrony/sources.d/nkp.sources # replace <NTP1>/<NTP2> first sudo systemctl restart chrony && sleep 10 && chronyc sources -v && chronyc tracking # "Leap status : Normal", internal sources only resolvectl status ¦ grep -E "DNS Servers¦DNS Domain" # from netplan; /etc/resolv.conf is the 127.0.0.53 stub - that is normal for h in pc.customer.local registry.customer.local nkp-bastion-01.customer.local; do printf "%-36s " "$h"; getent ahostsv4 "$h" ¦ awk '{print $1; exit}' ¦¦ echo "UNRESOLVED"; done dig -x <BASTION_IP> +short; dig -x <REGISTRY_IP> +short; dig -x <PC_IP> +short # Temporary /etc/hosts entries only when customer DNS is incomplete - remove before acceptance

## 4.6 Certificate trust — OS, Docker, and the PC bundle for NKP

Bastion — trust

# Customer root + intermediate chains land in /opt/nkp/pki (PEM). Ubuntu requires the .crt extension in the anchors directory. openssl x509 -in /opt/nkp/pki/customer-root-ca.crt -noout -subject -issuer -dates sudo install -m 0644 /opt/nkp/pki/customer-root-ca.crt /usr/local/share/ca-certificates/customer-root-ca.crt sudo install -m 0644 /opt/nkp/pki/customer-issuing-ca.crt /usr/local/share/ca-certificates/customer-issuing-ca.crt sudo update-ca-certificates # "2 added, 0 removed" ls -l /etc/ssl/certs ¦ grep -i customer # Docker trusts the registry CA per registry host REG=registry.customer.local sudo mkdir -p /etc/docker/certs.d/${REG} && sudo install -m 0644 /opt/nkp/pki/registry-ca-chain.crt /etc/docker/certs.d/${REG}/ca.crt # Verify both endpoints with the system trust (no -k, no --insecure) openssl s_client -connect ${REG}:443 -servername ${REG} -verify_return_error </dev/null 2>&1 ¦ grep "Verify return code" openssl s_client -connect pc.customer.local:9440 -servername pc.customer.local -verify_return_error </dev/null 2>&1 ¦ grep "Verify return code" # PC trust bundle for nkp --additional-trust-bundle (root + intermediate only; cloud-init limit 16 KB) cat /opt/nkp/pki/customer-issuing-ca.crt /opt/nkp/pki/customer-root-ca.crt > /opt/nkp/pki/customer-ca-bundle.pem wc -c /opt/nkp/pki/customer-ca-bundle.pem # < 16384

## 4.7 Kernel modules, sysctl, limits

Bastion — kernel

sudo install -m 0644 /opt/nkp/artifacts/config/nkp-bastion-modules.conf /etc/modules-load.d/nkp-bastion.conf sudo modprobe overlay && sudo modprobe br_netfilter && lsmod ¦ grep -E "^overlay¦^br_netfilter" sudo install -m 0644 /opt/nkp/artifacts/config/99-nkp-bastion.conf /etc/sysctl.d/99-nkp-bastion.conf sudo sysctl --system ¦ grep -E "ip_forward¦bridge-nf¦inotify" sudo install -m 0644 /opt/nkp/artifacts/config/nkp-bastion-limits.conf /etc/security/limits.d/nkp-bastion.conf ulimit -n # re-login to pick up 65536

## 4.8 Docker Engine from the local repository (pinned and held)

Bastion — Docker

source /opt/nkp/artifacts/config/VERSIONS.txt 2>/dev/null; echo "$DOCKER_VER" # or set DOCKER_VER from the kit manifest sudo -E apt-get install -y docker-ce="${DOCKER_VER}" docker-ce-cli="${DOCKER_VER}" containerd.io docker-buildx-plugin docker-compose-plugin sudo apt-mark hold docker-ce docker-ce-cli containerd.io # keep the validated build docker --version # >= 27.4.0 and exactly the pinned version # On Debian/Ubuntu the service starts during package installation - configure, validate, then RESTART sudo install -d -m 0755 /etc/docker sudo install -m 0644 /opt/nkp/artifacts/config/docker-daemon.json /etc/docker/daemon.json sudo vi /etc/docker/daemon.json # confirm default-address-pools overlaps nothing at THIS customer sudo dockerd --validate --config-file /etc/docker/daemon.json # "configuration OK" sudo systemctl enable docker && sudo systemctl restart docker && sudo systemctl --no-pager status docker ¦ head -5 sudo usermod -aG docker nkpadmin # log out and back in (or: newgrp docker) docker info --format 'Server {{.ServerVersion}} ¦ cgroup v{{.CgroupVersion}} ¦ {{.Driver}} ¦ root {{.DockerRootDir}}' docker network inspect bridge --format '{{(index .IPAM.Config 0).Subnet}}' # from your pool, e.g. 172.30.0.0/24 docker compose version; sudo aa-status ¦ grep -c docker-default # AppArmor profile for containers loaded # Do NOT run "docker run hello-world": it pulls from the Internet. The real test is 4.12 / Section 5.

Ubuntu specifics that bite Service starts on install: unlike RHEL, apt starts docker.service immediately, before daemon.json exists. Always systemctl restart docker after placing the file, and validate with dockerd --validate --config-file … first (the runbook's docker --validate is not a command). ufw and Docker: Docker programs iptables directly, so container ports published with -p bypass ufw rules. The bastion publishes nothing except Harbor 443 (if co-hosted); if the customer requires ufw to govern that too, add rules to the DOCKER-USER chain rather than fighting ufw. Held packages: apt-mark hold protects the validated Docker build from an accidental apt upgrade off a later kit; apt-mark showhold lists them.

## 4.9 CLI binaries — kubectl, helm, yq, nkp

Bastion — CLIs

cd /opt/nkp/artifacts/bin echo "$(cat kubectl.sha256) kubectl" ¦ sha256sum --check && sudo install -m 0755 kubectl /usr/local/bin/kubectl sha256sum --check helm-v3.*-linux-amd64.tar.gz.sha256sum && tar -xzf helm-v3.*-linux-amd64.tar.gz \ && sudo install -m 0755 linux-amd64/helm /usr/local/bin/helm && rm -rf linux-amd64 sha256sum --check yq.sha256 && sudo install -m 0755 yq /usr/local/bin/yq # optional # nkp: from the extracted bundle (Field Reference §4.1) - the only build that matches the images mkdir -p /opt/nkp/artifacts/nkp/extracted && tar -xzf /opt/nkp/artifacts/nkp/nkp-air-gapped-bundle_v2.18.x_linux_amd64.tar.gz -C /opt/nkp/artifacts/nkp/extracted sudo install -m 0755 "$(find /opt/nkp/artifacts/nkp/extracted -type f -name nkp ¦ head -1)" /usr/local/bin/nkp kubectl version --client; helm version --short; nkp version; yq --version kubectl completion bash ¦ sudo tee /etc/bash_completion.d/kubectl >/dev/null nkp completion bash 2>/dev/null ¦ sudo tee /etc/bash_completion.d/nkp >/dev/null ¦¦ true

## 4.10 Security baseline — AppArmor, ufw, SSH, proxy hygiene

Bastion — security

sudo aa-status ¦ head -3 # "apparmor module is loaded", profiles in enforce mode - keep it sudo ufw default deny incoming && sudo ufw default allow outgoing sudo ufw allow from <ADMIN_JUMP_CIDR> to any port 22 proto tcp # Harbor-on-bastion only: sudo ufw allow 443/tcp sudo ufw --force enable && sudo ufw status verbose # SSH hardening - only after key login as nkpadmin is proven from the jump host. Service name on Ubuntu is "ssh". sudo install -m 0600 /opt/nkp/artifacts/config/10-nkp-sshd.conf /etc/ssh/sshd_config.d/10-nkp-sshd.conf sudo rm -f /etc/ssh/sshd_config.d/50-cloud-init.conf # subiquity's "PasswordAuthentication yes" - would otherwise win sudo sshd -t && sudo systemctl restart ssh # Open a NEW ssh session from the jump host and confirm key login works and password login is refused # No proxy variables anywhere (they silently redirect nkp/docker/curl/apt) env ¦ grep -i proxy; sudo grep -ri proxy /etc/environment /etc/profile.d /etc/apt/apt.conf.d /etc/systemd/system/docker.service.d 2>/dev/null ¦¦ echo "no proxy configured" # Prove zero egress curl --head --connect-timeout 3 --max-time 5 https://download.docker.com && echo "FAIL: egress open" ¦¦ echo "PASS: egress blocked"

## 4.11 Optional — Harbor on the bastion (key points only; flow in runbook §8.4)

- sudo tar -xzf /opt/nkp/artifacts/harbor/harbor-offline-installer-v2.x.y.tgz -C /opt → /opt/harbor/harbor.yml: hostname: registry.customer.local, https.certificate/https.private_key = customer-issued server certificate and key (0600), data_volume: /srv/registry, strong harbor_admin_password (rotate after first login).

- Air gap: leave Trivy disabled, or --with-trivy with trivy.skip_update: true / trivy.offline_scan: true and a separately approved offline database process.

- sudo ./prepare then sudo ./install.sh; verify docker compose -f /opt/harbor/docker-compose.yml ps and curl --fail --cacert <ca> https://registry.customer.local/api/v2.0/ping. Add a systemd unit running docker compose -f /opt/harbor/docker-compose.yml up -d after docker.service.

- Create project nkp (private, quota ≥ 100 GB) and a robot account with push+pull — the credentials for nkp push bundle and --registry-mirror-*.

- sudo ufw allow 443/tcp and confirm every future NKP node can reach the bastion on 443 (runbook §3.2).

## 4.12 Load the bootstrap image, snapshot, hand over to Gate 2

Bastion — bootstrap image and baseline snapshot

cd /opt/nkp/artifacts/nkp/extracted/nkp-v2.18.x docker load --input ./konvoy-bootstrap-image-v2.18.x.tar docker images --format 'table {{.Repository}}\t{{.Tag}}\t{{.Size}}' ¦ grep -i bootstrap # Offline functional test of the whole runtime stack (no Prism Central needed) - same test as on the kit VM nkp create bootstrap 2>&1 ¦ tee /opt/nkp/logs/bootstrap-test.log # confirm flags with: nkp create bootstrap --help docker ps --format "table {{.Names}}\t{{.Status}}"; kubectl --kubeconfig ~/.kube/config get pods -A ¦ head nkp delete bootstrap df -h /opt/nkp /var/lib/docker; date -u +%FT%TZ ¦ tee /opt/nkp/evidence/bastion-baseline-utc.txt # Prism Central -> VM -> Actions -> Take Snapshot / Recovery Point: "nkp-bastion-01-baseline-<date>"

# 5. Gate 2 verification — bastion ready

| Check | Command | Expected |
| --- | --- | --- |
| OS baseline | lsb_release -ds; uname -r | Ubuntu 24.04.<x> LTS; kernel from the ISO (or the carried update) |
| Single apt source | grep -rh "^deb" /etc/apt/sources.list /etc/apt/sources.list.d/ 2>/dev/null | Only the file:/opt/nkp/artifacts/debs ./ line (plus the optional ISO line) |
| apt works offline | sudo apt-get update; apt-cache policy docker-ce | Completes with no network errors; candidate from file: |
| Held packages | apt-mark showhold | containerd.io docker-ce docker-ce-cli |
| Phone-home disabled | systemctl is-enabled apt-daily.timer snapd.service 2>/dev/null; ls /etc/cloud/cloud-init.disabled | disabled/masked; file exists |
| MAC / firewall | sudo aa-status ¦ head -1; sudo ufw status ¦ head -1 | "apparmor module is loaded"; "Status: active" |
| Services | systemctl is-active docker chrony ufw ssh | four × active |
| Time | chronyc tracking ¦ grep -E "Leap¦System time" | Leap status Normal; offset < 100 ms; internal source |
| DNS forward/reverse | getent ahostsv4 pc.customer.local registry.customer.local; dig -x <BASTION_IP> +short | All resolve; PTR returns the bastion FQDN |
| Registry TLS | openssl s_client -connect registry.customer.local:443 -servername registry.customer.local -verify_return_error </dev/null | Verify return code: 0 (ok) |
| PC TLS | same against pc.customer.local:9440 | 0 (ok) — or the --insecure exception recorded |
| Docker | docker info --format '{{.ServerVersion}} {{.CgroupVersion}} {{.Driver}}' | pinned version ≥ 27.4.0, 2, overlay2 |
| Docker pool | docker network inspect bridge --format '{{(index .IPAM.Config 0).Subnet}}' | Inside the non-overlapping pool you chose |
| Kernel | lsmod ¦ grep -E "overlay¦br_netfilter"; sysctl fs.inotify.max_user_watches | both modules; 524288 |
| Capacity | df -h /opt/nkp /var/lib/docker | ≥ 60 GB and ≥ 30 GB free respectively (after the bundle is extracted) |
| CLIs | nkp version; kubectl version --client; helm version --short | 2.18.x = bundle build; 1.35.x; v3.x |
| Bootstrap image | docker images ¦ grep -i bootstrap | Tag = the NKP build |
| Bootstrap functional (recommended) | nkp create bootstrap … nkp delete bootstrap | KinD cluster comes up and is deleted cleanly |
| Proxy hygiene | env ¦ grep -i proxy; ls /etc/apt/apt.conf.d ¦ grep -i proxy | empty |
| Egress | curl --head --connect-timeout 3 https://download.docker.com | fails |
| Admin access | new SSH session with key; ssh -o PubkeyAuthentication=no nkpadmin@<IP> | key works; password attempt refused |
| Recovery point | Prism Central snapshot exists | nkp-bastion-01-baseline-<date> |

# 6. Package reference and RHEL ↔ Ubuntu equivalents

| Item | Source | Purpose on the bastion |
| --- | --- | --- |
| bash-completion vim tmux git tar unzip zip xz-utils bzip2 rsync wget curl jq python3 tree | kit repo (Ubuntu main/universe) | Operator tooling; tmux for long nkp sessions; jq/python3 for the runbook's checks |
| dnsutils netcat-openbsd iputils-ping iputils-arping traceroute tcpdump net-tools socat ethtool lsof strace sysstat iotop | kit repo | DNS (dig), port tests (nc), arping -D for VIP checks, captures, MTU/route diagnosis |
| chrony openssl ca-certificates gnupg | kit repo | Internal NTP (replaces systemd-timesyncd); certificate inspection; trust store; key handling |
| lvm2 xfsprogs | base install / kit repo | Data disks |
| ufw apparmor-utils auditd | base install / kit repo | Host firewall; aa-status; audit trail for customer baselines |
| skopeo dpkg-dev apt-utils sosreport | kit repo | Push docker-archives without a daemon; rebuild the local repo on site; support bundles |
| docker-ce docker-ce-cli containerd.io docker-buildx-plugin docker-compose-plugin (+ resolved deps) | kit repo (Docker Ubuntu repo) | Bootstrap (KinD) runtime for nkp; Compose v2 for Harbor |
| Security updates for the base (optional) | kit repo (apt-get upgrade --download-only) | Patched bastion without Internet |
| kubectl 1.35.x / helm 3.x / yq | dl.k8s.io / get.helm.sh / GitHub | Cluster operations; chart inspection (not Helm 4) |
| nkp 2.18.x | inside the air-gapped bundle | The CLI matching the image set |
| NKP bundle + node OS image | Nutanix portal | Field Reference §3 |
| Harbor offline installer / validation images (optional) | GitHub / skopeo | Only without an enterprise registry / smoke test |
| Config files (§2.8) + autoinstall file | authored on the downloader | Deterministic, reviewable bastion build |

## 6.1 If you have the RHEL edition open next to this one

| Task | RHEL 9 edition | Ubuntu 24.04 edition |
| --- | --- | --- |
| Base package source on the bastion | Mounted DVD ISO = full BaseOS + AppStream | Only the .deb set you downloaded (ISO = base system only) |
| Download packages + dependencies | dnf download --resolve --alldeps against the ISO repos | apt-get install --download-only on an identical base (full closure: apt-cache depends --recurse) |
| Build the local repository | createrepo_c <dir> | dpkg-scanpackages + apt-ftparchive release + gpg sign (InRelease) |
| Repository definition | /etc/yum.repos.d/nkp-offline.repo (file://, gpgkey=) | /etc/apt/sources.list.d/nkp-offline.list (file: … ./, signed-by=) |
| Package integrity | Per-RPM signatures (rpm -K) | Signed index; per-package hashes in Packages |
| Disable online sources | Move other *.repo; disable subscription-manager plugin | Move ubuntu.sources; comment cdrom line; disable apt timers, unattended-upgrades, snapd, Pro client, cloud-init |
| Pin the runtime | Version in dnf download; installed from extras | Version in apt-get install pkg=ver; apt-mark hold |
| Trust store | /etc/pki/ca-trust/source/anchors/ + update-ca-trust | /usr/local/share/ca-certificates/*.crt + update-ca-certificates |
| MAC / firewall | SELinux enforcing / firewalld | AppArmor enforcing / ufw |
| Network / time | nmcli / chronyd (/etc/chrony.conf) | netplan YAML / chrony (/etc/chrony/sources.d/) |
| SSH service / admin group | sshd / wheel | ssh / sudo |
| Unattended install | kickstart (anaconda-ks.cfg) | autoinstall (autoinstall-user-data + cidata ISO) |
| Service start on package install | No — systemctl enable --now | Yes — configure then systemctl restart |

# 7. Variants and troubleshooting

## 7.1 Ubuntu 22.04 LTS delta

- ISO from releases.ubuntu.com/22.04/ (jammy); Docker version string ends in ~ubuntu.22.04~jammy; the repository codename in §2.4 resolves automatically.

- Online sources live in /etc/apt/sources.list (no deb822 ubuntu.sources): comment every deb line in §4.3.

- chrony 4.2 also supports sourcedir /etc/chrony/sources.d; netplan, ufw, AppArmor, ssh service name and cloud-init handling are unchanged.

- Ubuntu 22.04 is supported by NKP for custom node images only; for the bastion it is fine but 24.04 has the longer support horizon.

## 7.2 Podman instead of Docker Engine

NKP documents Podman 4.0+ as an alternative bootstrap runtime, and Ubuntu 24.04 ships Podman 4.9 in universe (podman, podman-docker for the docker command shim) — download it in §2.3 instead of the Docker set and no external repository is needed. Trade-offs: KinD under Podman must run rootful (sudo nkp … or a rootful socket), every public NKP air-gapped reference uses Docker, and Harbor requires Docker Compose. If the customer forbids Docker CE, install Podman from the kit and run the §2.9 offline bootstrap test with it before deciding.

## 7.3 Bastion from the NKP Ubuntu node image (not recommended)

The NKP Ubuntu 24.04 node image can be cloned into a bastion VM with a custom cloud-init user-data (a tip from partner deployments). It is CIS-hardened and carries containerd, but not Docker, and its package baseline is built for Kubernetes nodes, not for operators. Managing its packages offline still requires the repository from Part A, so it saves little; use the plain Ubuntu Server ISO unless the customer mandates the hardened node image for every VM.

## 7.4 Troubleshooting

| Symptom | Cause | Fix |
| --- | --- | --- |
| apt-get update: "Temporary failure resolving 'archive.ubuntu.com'" or hangs | An online source is still defined (ubuntu.sources, sources.list, a .list you forgot) | grep -rh "^deb\¦^URIs" /etc/apt/sources.list /etc/apt/sources.list.d/; move the offenders to /root/apt.disabled |
| "The following signatures couldn't be verified because the public key is not available: NO_PUBKEY …" | signed-by= path wrong, keyring not copied, or the index was re-signed with another key | Check /etc/apt/keyrings/nkp-kit.gpg; gpg --show-keys it; compare with the key that signed InRelease (gpg --verify InRelease) |
| "W: Download is performed unsandboxed as root as file … couldn't be accessed by user '_apt'" | Repository directory not readable/traversable by _apt | chmod o+x /opt/nkp /opt/nkp/artifacts; chmod -R o+rX /opt/nkp/artifacts/debs (a warning only — apt continues) |
| "E: Unable to locate package X" | Package not in the kit, or Packages not rebuilt after adding it | Check grep "^Package: X" /opt/nkp/artifacts/debs/Packages; add the .deb and rebuild/re-sign (§2.5) |
| "Depends: libfoo (>= x) but it is not going to be installed" / "unmet dependencies" | The bastion base differs from the downloader (updates during install, different ISO/edition) | Reinstall the downloader identically, or fetch the full closure (§2.3 callout); never mix .deb files from another release |
| "Version '5:…' for 'docker-ce' was not found" | Version string differs from what the kit contains | apt-cache madison docker-ce on the bastion shows the kit's version; use that |
| docker.service fails after placing daemon.json | JSON syntax or unknown key | sudo dockerd --validate --config-file /etc/docker/daemon.json; journalctl -u docker -n 50 |
| Docker runs but ignores daemon.json settings | Service was started by the package before the file existed | systemctl restart docker |
| docker load: "no space left on device" | /var/lib/docker not mounted (data landed on /) | df -h /var/lib/docker; fix fstab; systemctl restart docker |
| Bootstrap fails: "too many open files" / inotify errors | sysctl not applied | §4.7; sysctl --system; re-run |
| Bootstrap fails creating the kind network (IPv6) | ipv6.disable=1 on the kernel command line | Remove from GRUB (/etc/default/grub, update-grub) and reboot; dhcp6: false/accept-ra: false in netplan is fine |
| Containers cannot resolve DNS / bootstrap API unreachable | ufw or a customer nftables policy interfering with Docker's chains | sudo ufw status verbose; sudo iptables -L DOCKER-USER -n; test with docker run --rm --network host-free checks such as docker network inspect kind |
| Password SSH login still works after hardening | subiquity's 50-cloud-init.conf precedes a 50--numbered drop-in | Use 10-nkp-sshd.conf and delete 50-cloud-init.conf (§4.10); sudo sshd -T ¦ grep -i passwordauth |
| Interface renamed / static IP lost after reboot | cloud-init regenerated 50-cloud-init.yaml | touch /etc/cloud/cloud-init.disabled; keep one netplan file per NIC; netplan apply |
| netplan try reverted the configuration | No ENTER within 120 s, or console disconnected | Re-run from the Prism console; use netplan apply only after try succeeds once |
| needrestart prompts block scripted installs | Interactive service-restart dialog | export NEEDRESTART_MODE=a DEBIAN_FRONTEND=noninteractive |
| apt-get upgrade from the kit installed a new kernel | Errata set included linux-image-* | Reboot when /var/run/reboot-required exists; verify uname -r afterwards and re-run Gate 2 |
| AppArmor denials for containers | Profile conflicts from customer hardening | journalctl -k ¦ grep apparmor=\"DENIED\"; keep docker-default; do not disable AppArmor |
| nkp version works, nkp create bootstrap tries to pull from the Internet | Bootstrap image not loaded, or tag mismatch with the CLI build | docker images ¦ grep bootstrap; load the tar from the same bundle as the CLI |

## 7.5 Keeping the bastion patched later

Because the downloader has the identical base, the errata cycle is short: on the downloader run sudo apt-get update && sudo apt-get -y --download-only upgrade, copy the new .deb files into ~/kit/debs/pool, rebuild and re-sign the index (§2.5), carry only debs/ through the chain of custody, and on the bastion run sudo apt-get update && sudo apt-get -y upgrade (held Docker packages stay pinned; upgrade them deliberately with a new DOCKER_VER and a repeat of the offline bootstrap test). If the customer operates an internal Ubuntu mirror (e.g. Landscape or apt-mirror), point nkp-offline.list at it instead and retire the flat repository.